XJTLU Entrepreneur College (Taicang) Cover Sheet

| Module code and Title | MFE107TC – Thermodynamics and Fluid Mechanics |  |
| --- | --- | --- |
| School Title | School of Intelligent Manufacturing Ecosystem |  |
| Assignment Title | Thermodynamics and Fluid Mechanics Coursework |  |
| Submission Deadline |  |  |
| Final Word Count | - |  |
| If you agree to let the university use your work anonymously for teaching and learning purposes, please type “yes” here. |  |  |

I certify that I have read and understood the University’s Policy for dealing with Plagiarism, Collusion and the Fabrication of Data (available on Learning Mall Online). With reference to this policy I certify that:

- My work does not contain any instances of plagiarism and/or collusion.
My work does not contain any fabricated data.

By uploading my assignment onto Learning Mall Online, I formally declare that all of the above information is true to the best of my knowledge and belief.

| Scoring – For Tutor Use |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Student ID |  |  |  | 2472018 |  |  |
| Stage of Marking |  | Marker Code | Learning Outcomes Achieved （F/P/M/D） (please modify as appropriate) |  |  | Final Score |
|  |  |  | A | B | C |  |
| 1st Marker – red pen |  |  |  |  |  |  |
| Moderation – green pen |  | IM Initials | The original mark has been accepted by the moderator (please circle as appropriate): |  |  | Y / N |
|  |  |  | Data entry and score calculation have been checked by another tutor (please circle): |  |  | Y |
| 2nd Marker if needed – green pen |  |  |  |  |  |  |
| For Academic Office Use |  |  | Possible Academic Infringement (please tick as appropriate) |  |  |  |
| Date Received | Days late | Late Penalty | ☐ Category A |  | Total Academic Infringement Penalty (A,B, C, D, E, Please modify where necessary) _____________________ |  |
|  |  |  | ☐ Category B |  |  |  |
|  |  |  | ☐ Category C |  |  |  |
|  |  |  | ☐ Category D |  |  |  |
|  |  |  | ☐ Category E |  |  |  |

| Module | EXAMINER | DEPARTMENT | Email |
| --- | --- | --- | --- |
| MFE107TC | Long Huang | School of Intelligent Manufacturing Ecosystem | Long.Huang@xjtlu.edu.cn |

2nd SEMESTER 2024/25 Assignment

Undergraduate – Year 2

MFE107TC – Thermodynamics and Fluid Mechanics

Submission Deadline:  Apr. 11th, 2025. 12noon

INSTRUCTIONS

- This an individual assignment.

- Total marks available are 100. The contribution of this assignment to final mark is 65%.

- Answers to questions should be written/printed on clean A4 sheets.

- Marking criteria sheets  are provided as supplement documents.

- Only English solutions are accepted.

- The assignment report must be typed as an MS Word document. The interim presentation must be typed as a MS PowerPoint file. The deliverables must be submitted via Learning Mall Online to the correct drop box. Only electronic submission is accepted and no hard copy submission.

- All students must download their file and check that it is viewable after submission. Documents may become corrupted during the uploading process (e.g. due to slow internet connections). However, students themselves are responsible for submitting a functional and correct file for assessments.

| Student Name | Student ID |
| --- | --- |
| Juntong Fan | 2472018 |
| DEPARTMENT | Email |
| IME | Juntong.Fan24@student.xjtlu.edu.cn |

- Background

The last few years have seen an increased interest in China’s urban rail transit network, which by late 2024 encompassed 58 cities and 12,160.77 km, serving over 32.2 billion annual passengers [1]. The network’s vast geographical footprint spans a broad spectrum of thermal conditions—stretching from the frigid mid-temperate zone of Harbin to the tropical region of Sanya. Such diversity poses a serious challenge regarding the heterogeneous performance requirements of HVAC systems across coastal plains and high-altitude plateaus like Lhasa [1][5].

Within the context of southern China, HVAC systems are primarily dedicated to managing extreme latent and sensible heat loads to ensure thermal comfort [3][4]. In these regions, the synergy between high ambient temperature and humidity often pushes the human thermoregulatory system to its physical limits. Conversely, in severe cold regions, the issue of heating capacity and heat pump operational efficiency is central to system design [5]. Furthermore, against the backdrop of climate change, the necessity for adaptive, energy-efficient configurations that account for human thermal history has received significant research attention [6].

Transit demand has generated a wealth of data on acute spatial-temporal fluctuations. While passenger intensity in tier-one cities like Shenzhen (1.49 × 104 pass/km·day) is significantly elevated, emerging systems exhibit markedly lower levels [2]. These disparities, combined with temporal surges during public events, create highly transient and concentrated thermal loads [1][2].

Current ventilation strategies, however, often rely on steady-state assumptions that fail to account for the physiological heat storage (S) and metabolic decay (M(t)) during the transition from hot platforms to air-conditioned carriages. The limitations of these strategies are increasingly apparent, as they lack the efficient dynamic control needed to respond to real-time variations in moisture and the complex "comfort credit" provided by psychological expectancy [7]. Consequently, there is an urgent need for a dynamic modeling framework—such as the Expectancy-Adjusted Thermal Model (EATM)—that can bridge the gap between absolute physical loads and the transient sensory relief experienced by passengers in high-density underground environments.

This study was undertaken to develop a transient-adaptive thermodynamic model of passenger thermal comfort (EATM) specifically tailored for the high-density environment of urban rail transit.

The theoretical framework is rooted in Fanger’s "Comfort Equation" [22], further enhanced by the stepwise correction framework proposed by Shi et al. (2026) . To address the limitations of steady-state assumptions and characterize the physiological-psychological hysteresis, this research aims to evaluate the model's performance across operational intervals defined by the Subway Design Code (GB 50157) [23]. The specific objectives are as follows:

Characterize Dynamic Metabolic Trajectories (MAPS-Correction): To evaluate the metabolic adjustment for postural stability (MAPS) within the standard intervals of $1.2$ to $1.45$ met. This objective treats time ($t$) and initial activity intensity as primary independent variables to quantify the energy expenditure required against vehicle dynamics.

Quantify Crowd-Induced Radiative Shielding (MRT-Correction): To analyze the "view factor" ($F$) and mean radiant temperature ($T_r$) as a function of occupant density ($D$). By treating density (ranging from $0$ to $9$ persons/m²) as a key independent variable, the study simulates the "thermal shielding effect" where the micro-environment is dominated by human heat plumes rather than carriage boundaries.

Implement Expectancy-Adjusted Coupling (EATM): To quantify the "comfort credit" derived from the Thermal History Index ($THI$) based on standard temperature step-changes ($\Delta SET$). This involves establishing a Boundary Defense Mechanism that triggers a non-linear collapse of psychological expectancy when the skin wetness ($\omega$) exceeds the ISO 7730 threshold of $0.25$.

Dynamic Performance Benchmarking: To benchmark the transient $PMV_{EATM}$ trajectory against steady-state baselines (e.g., ASHRAE 55 / CBE Thermal Comfort Tool). By utilizing standard design ranges for humidity ($45\%-75\%$) and temperature ($24-30^{\circ}C$), this study seeks to identify the "Sensory Meltdown" points where adaptive thermal management must intervene to prioritize passenger-centric comfort over static HVAC output.

Figure 1: Methodological flowchart of the integrated PMV-PPD simulation model

- Problem Statement and Constraints

3.1 Physiological Module

This module defines the physical characteristics, metabolic pathways, and transient perceptual responses of the human subject, treating the passenger as an active heat source [47]. The theoretical framework underpinning this module consists of two distinct categories: fixed input constraints and derived computational parameters.

1. Metabolic Rate Baselines (M)

- Initial Metabolic Rate (Mplatform): Established at 1.6 met to integrate the average energy expenditure of intermittent movement and queuing periods on the platform [14].

- Steady-state Metabolic Rate (Mtrain): Set at 1.2 met, representing a standard sedentary standing position within the carriage [13].

2. Time Constants (τ)

- Physiological Balance Constant τ): Established at 900 s (15 minutes) to characterize the thermal decay period of heat storage when transitioning from an overheated state to a conditioned environment [14].

- Psychological Expectancy Decay (τexp): Adjusted to 72 s to assess the extent to which psychological perception and rapid metabolic shifts occur during high-paced transit transitions.

3. Expectancy and Defense Parameters

- Linear Expectancy Gain (G): Set at 0.303 to serve as the base weight for expectancy gain, maintaining logical consistency with classic thermodynamic scales [7][29][31].

- Sensory Breakdown Coefficient (k): Defined as 44 for high-fidelity simulations (academic research) or 20–30 for stable engineering control, representing the "Cliff Effect" [14][27].

4. Physiological Thresholds

- Critical Skin Wettedness (ωcrit): Defined as 0.25, consistent with ISO 7730 standards as the threshold where sticky discomfort occurs and psychological compensation fails [22][31][48].

- Typical Passenger Profile: Mass (mp) is 70 kg, height (lp) is 175 cm, and Initial Heat Storage (Sinitial) is approximately 335 kJ [14][26][50].

B. Derived Parameters:

The following physical quantities will be quantitatively assessed using the dynamic equations described in the modeling approach:

- Dynamic Metabolic Trajectory (M(t)): The non-linear metabolic response as a function of exposure time t [34].

- Transient Heat Storage Rate (S(t)): The physiological value representing internal energy accumulation, where Lt≡St [48][50].

- Psychological Expectancy Offset (λ): The "comfort credit" deduction calculated based on the Thermal Humidity Index (THI) and the boundary defense factor (fdef).

- Boundary Defense Factor (fdef): A logistic regression output used to trigger the non-linear collapse of psychological compensation [29][40].

- Effective Body Surface Area (Σbsa): The normalized heat exchange area derived from the DuBois formula [22][26].

- Skin Wettedness (ω): A dimensionless saturation index integrating both sensible and latent perspiration pathways [14][31][48].

- Steepness Coefficient (k≥20): This represents the "Cliff Effect." Once the passenger begins to sweat profusely due to high density or humidity, the psychological offset undergoes a Non-linear Collapse, forcing the HVAC control logic to prioritize physical cooling over energy-saving "comfort credits" [14][27].

3.2 Radiative Module

This module characterizes the asymptotic saturation of radiative interactions and the "Thermal Shielding Effect" in high-density subway environments, where the micro-environment is dominated by human heat plumes rather than carriage boundaries.

1. Geometric and Occupancy Constraints

Occupant Density (D): Varies from 0 to 9 people/㎡, simulating conditions from off-peak to the  extreme peak-hour limits defined by GB 50157 [23].

Maximum View Factor (Fmax): Established at 0.85 (Theoretical Saturation Limit), representing the physical limit where the horizontal field is entirely engulfed by the "human heat plume" [25][65].

Interpersonal Saturation Rate (kp): Set at 0.35, governing the rate at which the interpersonal radiative field densifies [23][65].

Baseline Wall View Factor (Fwall,0): 0.45, reflecting the proportional weight of lateral surfaces within the underground geometry under empty conditions [25][65].

Wall Occlusion Decay Constant (kω): 0.40, physically justified by the asymmetry of occlusion where the line-of-sight to the wall is obstructed more rapidly than interpersonal saturation [23][62][64][65].

Material Emissivity: Fixed between 0.90 ~ 0.98 for both clothing and skin surfaces to account for long-wave radiation exchange [13][25].

B. Derived Parameters

The following radiative quantities are dynamically assessed based on the interaction between occupant density and surface temperatures:

Human-to-Human View Factor (Fp-p): A function characterizing the asymptotic saturation of radiative interactions between passengers [65].

Human-to-Wall View Factor (Fp-wall): Quantifies the systematic occlusion of the radiative pathway between a passenger and the vehicle's side walls [62][64][65].

Corrected Mean Radiant Temperature (Tmr'): Characterizes the effective radiant field by redefining surrounding occupants as independent long-wave radiation sources (Tcl) [68][74].

Radiative Locking Effect: In high-density subway carriages, a significant Radiative Locking Effect emerges due to local crowd clustering [63]. As noted regarding the physical obstruction phenomena described by Miyamoto et al. [64], high occupant density effectively severs the radiative view factor between the human body and the cooled HVAC boundary walls. According to the mean radiant temperature weighting rules established in ISO 7726 (GB/T 40233) [25], interpersonal radiation becomes the dominant heat exchange pathway as the human-to-human view factor ($F_{p-p}$) rises sharply. This elucidates the phenomenon observed by Zhang et al. (2025) in numerical simulations: within dense crowds, the micro-environmental temperature in close proximity to the human body remains persistently high and cannot be effectively cooled, even despite the operation of vehicle air conditioning systems [62].

Boundary Wall Temperature (Twall,i): Dynamically estimated via a 1D Thermal Resistance Network to balance internal conduction with external environmental conditions (Text) [22][62][65].

3.3 Environmental Module Constraints

The environmental module defines the thermodynamic "ceiling" of the cabin and the systematic degradation of the passenger's thermal barrier under high-density conditions.

1. Microclimate Boundary Constraints

- Ambient Temperature (Ta): Established within the standard design range of 24∼30∘C to simulate varied operational conditions [20][23][60][74].

- Relative Humidity (RH): Defined within a standard range of $45% ~ 75% [9][23].

- Nominal Air Velocity (V): Baseline macro-ventilation flow provided by the HVAC system, typically ranging from to 0.5 m/s [27][76].

- Moisture Emission Linkage (η): Fixed at 0.022 to account for the mass fraction of water vapor in exhaled breath [25][74].

- Numerical Stability Constant (ϵ): Set at 0.001 m/s to eliminate mathematical singularities at zero air velocity and characterize the transition to natural convection [73].

2. Insulation and Shielding Parameters

- Initial Clothing Insulation (Icl,0): Adjusted by season, typically 0.5 clo for summer and 1.0 clo for winter [60].

- Lower Bound Protection (Insulation):

fwet (Moisture): A minimum protection bound of $0.6$ is enforced, ensuring that intrinsic

fiber resistance persists even when garments are fully saturated [14][22].

fclo (Compression): A truncation limit of $0.7$ is established to prevent a physically absurd

state of zero resistance under extreme crowd pressure [13][74].

- Flow Field Attenuation Coefficient (γ): Quantifies the physical resistance provided by the crowd to the macro-ventilation flow [76].

B. Derived Parameters

The following quantities are assessed to map the macro-environment onto the passenger's local micro-environment:

- Local Air Velocity (Vlocal): The attenuated velocity reaching the body surface, determined by the interaction between nominal velocity and occupant density (D) [34][[55]74].

- Effective Convective Coefficient (hcv,eff): Derived from the power-law relationship 8.3⋅Vlocal0.6 to quantify real-time convective heat flux [8][27][74].

- Localized Vapor Pressure (pvap,local): Characterizes the "Latent Heat Barrier" by accounting for moisture spikes from collective respiration in crowded carriages [31][74].

- Clo-Compression Function (fclo): Models the reduction of the "Microclimate Air Gap" resulting from physical contact between passengers [13][34][56][74].

- Local Convective Shielding Factor (fvent): A scaling operator that accounts for the physical mass of occupants acting as obstacles to airflow [32][63][74].

- Modeling Approach

Physiological and environmental interaction heat balance equation:

M - W = (R + C + Esk) + (Cre + Ere) + S

The evaluation of human thermal comfort is fundamentally governed by the First Law of Thermodynamics, which dictates the conservation of energy between the human body and its immediate environment. Drawing upon the seminal framework established by Fanger (1973) [22],This comprehensive expression accounts for the net metabolic heat production (M-W), sensible heat exchange via radiation (R) and convection (C), total latent heat loss through the skin (Esk), and respiratory heat exchange (Cre , Ere). Within this framework, the term S represents the rate of heat storage; while Fanger’s steady-state PMV model identifies optimal comfort at S = 0 , the inclusion of S is critical for analyzing thermal transients and the dynamic load on the human thermoregulatory system.

Sensible Respiratory Heat Loss (Cre ):  Cre = 0.0014 ∙M  ∙ (34 - Ta)

This expression describes the sensible heat loss incurred by heating inhaled air from the ambient temperature (Ta) to the body's internal temperature[9][27].

Latent Respiratory Heat Loss (Ere): Ere = 1.7 × 10-5 ∙ M  ∙ (5867 - Pa)

This formula addresses the issue of  evaporative heat dissipation resulting from the humidification of inhaled air within the lungs, where Pa represents the ambient vapor pressure. While some recent studies suggest a more precise coefficient of  1.73 × 10-5[9], it is generally accepted  that the value  1.7 × 10-5  remains the standard for ISO 7730 compliance[27].

In the literature on thermal comfort, the comprehensive PMV formula is established by calculating the discrepancy between internal heat production (M-W) and various pathways of heat loss, which are explicitly incorporated into the main equation. The justification for a decoupled modeling approach is drawn from the underlying logic of the ISO 7730 standard[27]. The theoretical framework underpinning the standard’s computer program provides a basis for treating different heat loss pathways as discrete items during calculation. By isolating respiratory dissipation from skin-level convection and radiation, the model confirms the centrality of independent pathway analysis in establishing thermal equilibrium.

In the context of overcrowded subway environments, this decoupling addresses the issue of the shrinking "driving potentials". The efficiency of respiratory heat loss is strictly defined by the gradients of ambient temperature (34 - Ta) and vapor pressure(5867 - Pa). As these micro-environmental factors rise concurrently in a crowded carriage, the mechanisms underlying heat dissipation are effectively blocked. This highlights the key differences between total thermal load and the perceived "stuffiness" that convective airflow alone cannot mitigate, as the physical limits of respiratory cooling are reached.

Dynamic Formulation of Transient Heat Storage S(t):  S(t) = Sinitial ∙ e-t/τ

Rather than treating heat storage (S) as a static value, this study proposes a series of solutions for defining it as a time-dependent function. This study is aimed at capturing the exponential decay of heat energy as a passenger transitions from a high-temperature exterior to a climate-controlled cabin. In this function, Sinitial represents the initial heat storage level upon entry (approximately 335kJ) .τ is the time constant reflecting the “15-20 minute" equilibrium window identified in recent research. From a physical perspective, this expression describes the physiological shift where S ≠ 0 during the initial 20 minutes, followed by a transition to S ≈ 0 as the body reaches a quasi-steady state [14].

Expectancy-Adjusted Thermal Model (EATM): PMVEATM = PMVdynamic  - λ(t,THI, ω)

The formulation of the Expectancy-Adjusted Thermal Model (EATM) is theoretically grounded in the proven limitations of the classic PMV index, which strictly assumes a steady-state equilibrium (S = 0) and fails to capture the dynamic thermal load in highly transient underground micro-climates[22][27][29]. The EATM's bifurcation into a dual-process framework—coupling thermophysiological inertia with instantaneous psychological adaptation—is directly supported by Goto et al., who demonstrated that while physiological heat balance requires 15–20 minutes to equilibrate, human thermal sensation responds almost instantaneously (within 1 minute) to environmental step-changes[14][29][38].

To mathematically bridge this perceptual-physiological misalignment, the EATM adopts a subtractive structural approach akin to recent hybrid comfort models[29]. The introduction of the Psychological Offset (λ), driven by the Thermal History Index (THI), quantifies the transient “comfort credit” (which can account for up to a 3–5°C tolerance shift in subway stations)[20][31][32]. Crucially, this psychological adaptation is governed by a Boundary Defense Mechanism defined by the Skin Wetness (ω).

This offset accurately isolates the psychological adaptation from the Dynamic Physical Base (PMVdynamic  ), provided that physiological boundaries—such as humidity-induced evaporative limits—are not breached [16][31][31].

Psychological Offset (λ): λ(t, THI, ω) = G ∙ THI(t) ∙fdef (ω)

The Psychological Offset (λ) is defined as a hybrid corrective term that quantifies the transient "comfort credit" experienced by passengers during environmental transitions. It is mathematically formulated as the product of Thermal History Intensity (THI) Linear Expectancy Gain (G) and a Non-linear Boundary Defense Filter (fdef).

Initially, the model acknowledges that passengers perceive temperature shifts through the lens of Thermal Alliesthesia rather than immediate physical loads [20][22][33] . By aligning the Linear Expectancy Gain (G)  with Fanger’s established value of 0.303 [22][29][31], the model allows a 3–5°C "comfort bonus" to translate into a unit shift in the predicted thermal sensation. This bonus remains transient, governed by the exponential decay term , which ensures the effect dissipates over a 15-minute "Expectancy Window" as the body approaches a quasi-steady state [14][48]. However, this psychological relief is subject to a Physiological Veto via the function to prevent sensory divergence. Once skin wettedness reaches the Critical Threshold (ωcri)[32][40]—a value rooted in ISO 7730 and Gagge’s research—the model triggers a "Cliff Effect" (k≥20) [14][27]. At this point, acute tactile discomfort from moisture and stickiness overrides any anticipatory relief, causing the psychological offset to undergo a Non-linear Collapse [16][29]. Consequently, the HVAC control logic is forced to pivot from energy-saving "comfort credits" to prioritizing rigorous physical cooling to maintain passenger well-being in high-density or high-humidity conditions [8][20].

Boundary Defense Function (fdef):  fdef =  11+expk∙ω-ωcri

The Boundary Defense Function ((fdef) is a non-linear, logistic "cutoff" mechanism within the Expectancy-Adjusted Thermal Model (EATM) [14][16][50]. Acting as a Physiological Veto, its primary purpose is to trigger a rapid collapse of psychological expectancy when physiological thermal stress exceeds a critical threshold, ensuring the model remains anchored to sensory reality [31][32]. This specific definition is necessitated by the fundamental principle that psychological adaptation has finite limits—essentially, "psychology cannot override physiology" in high-density thermal environments [12][22][34].

Skin Wettedness (ω)：ω = EskEmax =  Ediff + Ersw Emax

This non-dimensional parameter quantifies the proportion of the skin surface required to be covered by moisture to maintain homeothermy [31][38]. In this architecture, the numerator represents the Total Evaporative Heat Loss (Esk), which is decoupled into two distinct physiological streams: the Insensible Evaporation Heat Loss (Ediff) and the Active Sweat Evaporation Heat Loss (Ersw). The former establishes a basal "physical floor" of moisture—approximately 6% of the total evaporative demand—stemming from the continuous diffusion of water through the dermal layers, while the latter represents the regulatory glandular response to thermal stress or escalating metabolic intensity. By summing these components, the model captures the cumulative latent heat flux required by the passenger to counteract the dynamic thermal load.

Furthermore, the denominator, Maximum Evaporative Capacity (Emax), functions as the "atmospheric ceiling" or the absolute thermodynamic bottleneck of the environment. In high-density subway micro-climates, this ceiling is frequently suppressed due to the "thermal shielding" effect of nearby occupants and restricted airflow, which severely inhibits the convective heat transfer coefficient (hc) and, consequently, the evaporative heat transfer coefficient (he). When the biological demand for cooling Esk) approaches this physical limit, the ratio ω escalates toward its asymptotic saturation limit of 1.0 [31].

The cybernetic logic of this formula is central to understanding the transition into the 15-minute "expectancy window" [31][32]. While the Psychological Offset (λ) provides immediate relief, the skin wettedness trajectory provides the "sensory reality" of the environment. Because skin wettedness above a specific critical threshold triggers acute non-thermal discomfort—such as stickiness and clothing adherence—it acts as a powerful counter-balance to the anticipatory effect. By integrating these high-density variables, the EATM ensures that the final comfort prediction rigorously reflects the sensory discomfort caused by restricted evaporation in crowded cabins, even when the ambient air temperature appears ostensibly cool [32].

Refined Skin Wettedness: ωsk = EskEmax

This specialized formulation provides a detailed account of the skin wettedness trajectory by isolating the baseline physiological state from regulatory sweat flux [22]. The constant 0.06 establishes the Basal Skin Wettedness, representing an irreducible "physical floor" where moisture continuously diffuses through the dermal layers regardless of thermal stress [31]. Conversely, the coefficient 0.94 scales the Active Sweat Evaporation Heat Loss (Qsw) against the Maximum Evaporative Capacity (Emax), which functions as the thermodynamic "ceiling" of the micro-environment.

By integrating these specific components, the model clarifies the role of moisture saturation in counter-balancing the initial psychological relief often attributed to thermal expectations [28][31]. This mechanism ensures that the 15-minute "expectancy window" transition remains anchored to the sensory reality of the cabin, where the escalation of ωsk toward its asymptotic limit eventually offsets the anticipatory effect and temporal alliesthesia as exposure time increases [12][14][33].

Dynamic Physical Base Formula (PMVdynamic  ):  PMVdynamic(t) = [0.30 ∙e-0.036∙Mt3+ 0.028]∙L(t)

The core of this EATM framework, the Dynamic Physical Base (PMVdynamic), establishes the "physiological truth" by decoupling absolute thermal load from a metabolic-dependent scaling factor derived from ISO 7730 regression parameters [16][13]. This scaling mechanism utilizes parameters established by P.O. Fanger through the statistical analysis of over 1,300 subjects, where the constant 0.303 functions as the primary sensitivity coefficient mapping physical thermal imbalance (W/m2) onto the subjective sensation scale[22]. Central to this formulation is the exponential term, e-0.036∙Mt, which formalizes the non-linear nexus between metabolic intensity and thermal sensitivity[13][14]. This structure effectively quantifies the physiological saturation effect: as a passenger’s metabolic activity escalates—such as during the rapid transition from locomotion to stationary standing—the neurological sensitivity to thermal load gradually diminishes. Combined with the asymptotic limit of 0.028, this ensures the model maintains mathematical robustness and prevents sensory divergence even under extreme metabolic conditions [27][29].

Dynamic Thermal Load L(t): L(t) = S(t) =  M(t) – W - (R + C + Esk + Cre + Ere)

The multiplicative relationship between this sensitivity bracket and the Dynamic Thermal Load (L(t)) is fundamental to the model’s cybernetic logic. In this structure, L(t) quantifies the absolute physical energy imbalance in real-time by integrating various microclimate variables and high-density corrections—related to evaporative fluxes and clothing resistance—that modulate the overall PMVdynamic trajectory[13]. The sensitivity bracket acts as a dynamic gain factor that scales this raw physical load according to the body's current physiological capacity to perceive stress. Because the metabolic rate M(t) itself follows a first-order exponential decay rather than changing instantaneously, this configuration ensures the model rigorously reflects the delayed physiological thermoregulatory response of the passenger rather than a static snapshot [12][14]. While L(t) traditionally represents the "load" required to maintain comfort, this research attempts to establish the connection between the calculated thermal load and the transient heat storage rate S(t). Specifically, within the dynamic regime, this study establishes the identity L(t) ≡ S(t), identifying S(t) as the primary physiological truth representing internal energy accumulation.

Complementing this objective physical base is the Psychological Offset (λ), a subtractive term designed to quantify the Anticipatory Effect or the immediate sensory relief experienced upon entering a conditioned space [22]. Driven by the Thermal History Index (THI), this offset acts as a compensatory buffer that "discounts" the objective thermal stress by acknowledging the intrinsic psychological expectation of comfort following an environment transition[16][29]. This synergy between  PMVdynamic  and λ identifies a strategic 15-minute "expectancy window" during the initial stages of transit [12][14]. By providing a high-fidelity tool that accounts for this perceptual-physiological misalignment, the EATM enables HVAC systems to optimize energy consumption and mitigate overcooling without compromising the perceived well-being of the passengers.

Thermal History Intensity (THI(t)): THI(t) = ΔSETn-1  ∙αn-1   ∙ e-t/τexp

The Thermal History Intensity (THI(t)) index quantifies the integrated psychological and physiological impact of prior thermal exposure triggered during the transition between adjacent spaces. The subscript n-1 serves as a technical anchor linking the passenger's current perception at location n to their immediately preceding environment. This is grounded in the "Neighborhood Effect," where empirical evidence confirms that while the most recent transition significantly modulates current sensation, the influence of earlier stages (e.g., n-2) decays to insignificance during continuous transit [32].

Coupled Magnitude (ΔSETn-1): ΔSETn-1 = SETn - SETn-1

This term represents the instantaneous temperature gradient between the current (n) and previous (n-1) space. It utilizes the Standard Effective Temperature (SET) algorithm, a pre-existing heat-balance solver derived from the Gagge Two-Node model. The calculation of  SET is governed by six fundamental factors: ambient temperature (Ta), relative humidity (RH), mean radiant temperature (Tmr), air velocity (Va), metabolic rate (M), and clothing insulation (Icl). By using SET, the model captures the integrated "thermal shock" resulting from both environmental changes and metabolic shifts.

Following the "neighborhood effect" identified by Shi et al. (2026), the subscript n-1 anchors the calculation to the immediately preceding environment. Empirical evidence shows that while the most recent transition significantly modulates current sensation, the influence of earlier stages (e.g., n-2) decays to an insignificant level [32].

Adaptation Coefficient (α)：

The coefficient α functions as a continuous Adaptation Filter that quantifies the retention proportion of the psychological expectation. This continuous parameterization replaces discrete stepping to ensure numerical stability for gradient-based optimization. The model employs Asymmetric Adaptation logic based on the direction of the step-change:

- Initial State (n=1): At the first entry point, α0=  1, allowing the psychological potential to be fully triggered.

- Cooling Step (ΔSET< 0):α is assigned a higher default (e.g., 0.6), reflecting a slower habituation to "relief" and a higher retention of the comfort credit.

- Warming Step ΔSET > 0): α is assigned a lower default (e.g., 0.4), capturing the rapid "desensitization" to thermal stress as psychological compensation collapses.

Temporal Dissipation (Φ(t)): Φ(t) =  e-t/τexp

This exponential decay factor models the Habituation process over time (t). It defines the 15-minute "Expectancy Window," where the initial psychological offset is strongest at entry and gradually dissipates as the passenger’s physiological state approaches a quasi-steady state equilibrium (τexp ≈ 900s) [32].

Metabolic Trajectory (M(t)):  M(t) = (Mplatform - Mtrain) ∙ e-t/Tphys} + Mtrain

The formulation of the Metabolic Trajectory, facilitates the shift from locomotive thermogenesis on the platform to stationary equilibrium within the carriage. Traditional thermal comfort modeling has historically relied on simplified "activity diaries" that offer merely a static snapshot, frequently failing to account for real-time dynamic activity changes and transient physiological states [14][32][38]. By contrast, the continuous formulation of ensures that the physiological "residual heat" is explicitly integrated into the internal heat storage term. This formulation captures the intrinsic biological delay in human thermoregulation [32], directly reflecting the physiological reality that it takes approximately 15 to 20 minutes for the human body to match heat dissipation with heat production and re-establish a new physiological steady-state following a step-change in activity [31][32].

To ensure the theoretical robustness of this framework, the model incorporates a strict boundary check. It confirms that as t→∞, the metabolic rate converges to the baseline stationary state Mtrain. At this limit, the model successfully reverts to the classic steady-state Fanger's comfort equation underpinning the ISO 7730 standard [32][38], which is strictly derived for and exclusively applicable to steady-state thermal conditions [14][27][22].

Furthermore, a gradient check performed on the derivative serves to quantify the "heat shielding" effect [33]. Utilizing derivative mechanisms to evaluate transient conditions is a proven approach in Dynamic Thermal Load (DTL) methodologies, which model time-varying characteristics of human thermal load [29]. In this context, the gradient mathematically demonstrates how the rate of metabolic decline is suppressed as occupant density increases. By rigorously accounting for these high-density variables, the Adaptive Metabolic Engine transcends traditional static evaluations. Rather than relying on a static snapshot, this high-fidelity mechanism explicitly reflects the delayed physiological response of passengers adapting to real-world, transient vehicular environments [14][33].

Human-to-Human View Factor (Fp-p):  Fp-p = Fmax  ∙  (1 - e-kp ∙D)

This function characterizes the asymptotic saturation of radiative interactions between passengers in high-density environments. As occupant density (D) escalates, the horizontal radiative field of a target passenger is progressively occupied by the physical boundaries of surrounding occupants. The formulation utilizes a (1 - e-x) structure to accurately reflect the overlapping occlusion effect derived from probability theory: at low densities, the increase in the view factor is nearly linear; however, at extreme densities, additional passengers are increasingly "hidden" behind the first layer of the crowd, causing the view factor to converge towards its geometric limit, Fmax.

Scientific Validation of Parameters [65]:

Human-to-Wall View Factor ( Fp-wall):  Fp-wall =  Fwall,0 ∙ e-kω ∙D

Definition and Physical Interpretation: This function defines the systematic occlusion of the radiative pathway between a passenger and the vehicle's side walls [62]. It quantifies the Thermal Shielding Effect, wherein the radiative heat flux toward the cooler boundary surfaces (e.g., windows and air-conditioned panels) is intercepted by the physical mass of other passengers. The exponential decay function provides a high-fidelity representation of how the "line-of-sight" to the environment is filtered and ultimately severed by successive layers of the crowd [64][65].

Corrected Mean Radiant Temperature (Tmr'): 
Tmr'=Fp-p⋅Tcl+273.154+Fp-wall⋅Twall,i+273.154+1-Fp-p-Fp-wall⋅Ta+273.1540.25-273.15

This formulation characterizes the asymptotic saturation of radiative interactions by redefining surrounding occupants as independent long-wave radiation sources [8][25][50][52][68][74].

Occupant Heat Source (Tcl)

The model links Tmr' to the clothing surface temperature (Tcl), which is derived from the physiological module's iterative solver. Drawing on the work of density-dependent configuration, the interpersonal view factor (Fp-p) forces the radiant field to converge toward Tcl as occupant density (D) escalates, thereby characterizing the "Radiative Locking" effect [74]. This mechanism demonstrates how, in high-density scenarios, the clothing surface temperature of surrounding passengers supersedes cooler boundary surfaces as the dominant radiative heat source [68].

Boundary Wall Temperature (Twall,i)

In an attempt to avoid reliance on static assumptions, the internal boundary temperature (Twall,i) is dynamically estimated via a 1D Thermal Resistance Network. This value is linked to the external ambient temperature (Text) to provide a theoretical basis for background radiation corrections:

Twall,i=Text-Text-Ta⋅Rext+RwallRtotal

This process is aimed at balancing internal heat conduction with external convective and radiative loss to provide a high-fidelity background field [13][22][62][65][68].

This formulation characterizes the asymptotic saturation of radiative interactions in restricted, high-occupancy spaces where traditional calculations are insufficient [30][59][62]. By isolating the human heat plume from the vehicle's thermal boundaries, the model accurately reflects why passengers experience extreme "stuffiness" even when the HVAC air temperature (Ta) is low [74].

As occupant density (D) increases, the horizontal radiative field is progressively occupied by the physical boundaries of others [32][53][55]. This reconstruction of the radiative pathway causes the total heat load to increase twofold or more [31], forcing the skin wettedness (ω) to break the 0.25 critical threshold [22][32][52]. Once the boundary defense mechanism (fdef) is triggered, the mechanisms underlying the psychological offset (λ) undergo a nonlinear collapse [16][40][56]. This reflects a transition where the "line-of-sight" to the cooling environment is effectively severed, resulting in a state of extreme discomfort [8][12][22][65].

Clo-Compression Function (fclo): Icl,final=Icl,0⋅fcloD⋅fwetω

This function addresses the issue of the systematic degradation of thermal insulation properties (Icl) resulting from high-density occupant compression in restricted transit environments [23][34][60][61]. The formulation characterizes the reduction of the "Microclimate Air Gap"—the layer of stationary air entrapped between the fabric and skin [27][47][56]. As occupant density (D) escalates beyond the key ergonomic turning point, physical contact between passengers forces this air to be "pumped" out. This mechanism is inextricably intertwined with the "Wet-Clo Effect," where moisture from rising sweat levels replaces air within the textile fibers, causing a systematic drop in conductive resistance [14][31][52][65].  This integrated closure characterizes the 'dual strike' on the passenger’s thermal barrier: the external mechanical load from crowd compression (fclo) [34], and the internal physiological moisture from active sweating (fwet) [31][50][65].

The introduction of fcloD provides precision and clarity regarding the "physical reality" feedback loop in the EATM framework. As the effective insulation Icl,corr decreases, the balance of dry heat exchange is disrupted [14][27][29][30][50]. This directly drives the skin wettedness (ω) to increase twofold or more, forcing it to break the 0.25 critical threshold [31][32][59]. Once the boundary defense mechanism (fdef) is triggered, the mechanisms underlying the psychological offset (λ) undergo a nonlinear collapse [40][60]. This results in a transition to extreme discomfort as the passenger's primary thermal barrier fails under the mechanical load of the crowd [22][29][36][65].

Moisture-dependent Insulation Decay Factor (fwet): fwetω=min1.0,max0.6,exp-αde⋅ω-ωcrit

This function addresses the issue of the non-linear degradation of clothing insulation properties (Icl) resulting from moisture accumulation within the textile matrix [14][50][56]. It characterizes the transition from "dry" to "wet" states, specifically correcting for the "Wet-Clo Effect" observed in high-density restricted environments [20][59][60]. The underlying mechanisms involve the displacement of stationary air by sweat, which possesses significantly higher thermal conductivity [22][47]. Unlike linear models, this formulation accounts for the asymptotic saturation of moisture impact, ensuring that the thermal barrier does not undergo a physically absurd collapse at low levels of skin wettedness (ω)  [31][65].

The inclusion of fwetω provides precision and clarity regarding the physiological and physical coupling within the EATM framework. As skin wettednessωescalates beyond the threshold, the resulting drop in thermal resistance breaks the equilibrium of dry heat exchange. This creates a positive feedback loop that causes the total heat load Lt to increase twofold or more, ultimately triggering the boundary defense mechanism (fdef) [31]. The mechanisms underlying this transition drive the nonlinear collapse of the psychological offset λ[29][40], accurately reflecting the rapid deterioration of passenger comfort once the "Wet-Clo" state is achieved [60][65].

Local Convective Shielding Factor (fvent):

fventV,D=11+γ⋅D⋅V+ϵ-1

This function addresses the issue of the systematic reduction in convective cooling efficiency within high-density restricted environments [62][63]. It characterizes the "Shielding Effect," wherein the physical mass of occupants acts as a series of obstacles that intercept and dissipate airflow momentum. The underlying mechanisms involve the attenuation of the local velocity field, which significantly reduces the convective heat flux reaching the target passenger's body surface. Unlike models that assume uniform air distribution, this formulation accounts for the asymptotic saturation of airflow blockage , ensuring that the convective coupling remains physically robust even as occupant density (D) approaches extreme levels [65].

Scientific Validation of Parameters:

- Flow Field Attenuation Coefficient (γ): This parameter is aimed at quantifying the physical resistance provided by the crowd to the macro-ventilation flow. It provides a detailed account of how effectively the interpersonal spacing filters the air velocity (V) before it reaches the target, a process critical to determining the heat exchange balance [65].

- Numerical Stability Constant (ϵ= 0.001): This constant is defined as a safeguard to eliminate mathematical singularities when $V=0$. Specifically, it serves to provide a theoretical basis for the transition to natural convection under stagnant air conditions [65].

- Effective Coefficient (hcv,eff): This variable is linked to the actual thermal dissipation capacity of the passenger, where the convective heat loss coefficient is typically derived from equations like hcv=8.3×V0.6. This ensures precision and clarity in calculating the dry heat exchange balance [65].

Modeling Implications: The inclusion of fvent provides an overview of the dynamic aerodynamic-thermal coupling within the EATM framework. As occupant density (D) escalates beyond the threshold, the resulting drop in convective heat transfer breaks the equilibrium of thermal dissipation [22], leading to a reduced proportion of convective heat loss in the total sensible heat loss—dropping from 46.1% to 45.4% in extreme cases.

This creates a bottleneck that causes the total heat load (L(t)) to undergo a significant redistribution, forcing the skin wettedness (ω) to rapidly breach the 0.25 critical threshold as sweat evaporation becomes the dominant mechanism. The mechanisms underlying this convective suppression drive the nonlinear collapse of the psychological offset (λ), accurately reflecting the transition to extreme thermal distress as the passenger's "convective line-of-sight" to the HVAC cooling source is effectively severed [62][65].

Effective Convective Heat Transfer Coefficient (hcv,eff):

hcv,eff=hcv,ref⋅fventV,D

This formulation addresses the issue of the discrepancy between theoretical heat loss and actual dissipation in non-uniform, high-density environments [62]. hcv,eff characterizes the effective thermal coupling between the passenger's exterior surface and the attenuated airflow. While fvent accounts for the physical obstruction,  hcv,ref serves as the actual interface variable that quantifies the real-time convective heat flux. This approach ensures precision and clarity in defining the thermal boundary conditions, as the coefficient is inextricably intertwined with the geometric occlusion of the surrounding crowd [63][65].

The calculation of hcv,ref provides a detailed account of the dry heat exchange imbalance within the EATM framework. It acts as the primary driver for the Total Heat Load (L(t)). As hcv,ref drops due to rising occupant density (D) , it triggers a surge in skin wettedness (ω), ultimately leading to the nonlinear collapse of the psychological offset (λ). This ensures that the model reflects the "convective bottleneck" experienced during extreme peak-hour overcrowding [62][65].

Local Air Velocity (Vlocal):

VlocalD=V⋅fventV,D+ϵ≈V⋅e-γ⋅D+ ε

hcv,eff=8.3⋅VlocalD0.6

This function set provides a detailed account of the heat exchange imbalance within the EATM framework. It accurately reflects how the thermal boundary conditions are inextricably intertwined with the geometric occlusion of the surrounding crowd. Taking these factors into account, the model demonstrates why maintaining high HVAC output often fails to provide relief in extreme peak-hour overcrowding due to the physical severance of the convective pathway [74].

Local Velocity Mapping (Vlocal)

The model links the local air velocity Vlocal to the nominal HVAC air velocity (V) and occupant density (D). As  D increases, the physical mass of occupants acts as a series of obstacles that intercept and dissipate airflow momentum, causing Vlocal to decay exponentially [76]. This approach ensures precision and clarity by identifying the key factors behind local airflow attenuation, where the flow field decay coefficient (γ) directly converts density into local velocity loss.

Convective Coupling (hcv,eff)

The calculation of (hcv,eff) derived from the power-law relationship 8.3⋅VlocalD0.6, which is consistent with ISO 7730 standards for human convective heat loss [68]. By utilizing Vlocal instead of the macro-average V, the variable serves as the actual interface variable that quantifies the real-time convective heat flux [74]. This formulation highlights the key differences between theoretical cooling capacity and the actual heat dissipation obstacles encountered in the center of a crowded carriage where air velocity effectively drops to zero.

Sensory Breakdown Trigger

The reduction in hcv,eff is viewed as the primary physical driver for the escalation of skin wettedness (ω). When occupant density D causes the dry heat exchange to drop significantly, the human thermoregulatory system is forced to rely almost exclusively on evaporative cooling. This imbalance triggers a surge in ω, eventually breaking the 0.25 critical threshold and leading to the nonlinear collapse of the psychological offset (λ) [22].

Definition of Body Surface Area (BSA):  
Σbsa=0.007184⋅mp0.425⋅lp0.725

For the purpose of this study , Body Surface Area (Σbsa) is defined as the total effective heat exchange surface of a human subject. The theoretical framework underpinning this definition is the DuBois model, a widely accepted standard used to normalize metabolic heat production into heat flux per unit area (W/m2) [14][22][34][50].

The decision to utilize this specific model is driven by the need to establish a consistent physical boundary for thermodynamic (heat exchange) modeling. This approach is significant for the following reasons:

- Normalization of Physiological Data: The function provides a theoretical basis for converting total metabolic heat production into a standardized heat flux, which is essential for accurate thermal calculations [34].

- Addressing Individual Variability: By incorporating mass and height, this formula addresses the issue of thermal load imbalances caused by individual physiological differences [14].

- Determining System Capacity: Within the field of urban transit modeling, this parameter is used to identify the main factors behind the maximum thermal load in high-density scenarios, such as crowded subway carriages [8][20][34][62].

Vapor Pressure  (pvap): pvap=ϕrel⋅exp18.6686-4030.183Text+235

Consistent with ISO 7726 standards [25], this function serves as the core input for calculating skin wettedness (ω) [14][29][31][48], thereby addressing the issue of latent heat regulation [8][22][20]. The application of this model is critical for shedding light on how local pvap levels often rise due to the accumulation of moisture from collective respiration in high-density scenarios, such as crowded subway carriages [7][34][62]. Furthermore, by identifying the key variables affecting the evaporative cooling path (Esk) [50], the formula highlights the key differences between theoretical cooling capacity and actual heat dissipation while examining the degree to which the humidity gradient is suppressed [31]. Taking these factors into account , the pvap value is viewed as the fundamental determinant of the "latent heat barrier" in thermodynamic modeling [12][29].

Temperature of the Outer Clothing Surface (Tcl):

Tcl=35.7-0.028⋅M-W-Icl,r⋅{hc⋅Tcl-Text+hr⋅Tcl-Tmr}

In light of high-density subway scenarios, this model is essential for identifying the key factors behind acute thermal discomfort. A number of studies have demonstrated that "fabric compression" and activity-induced air displacement significantly lowerIcl,r, while high passenger flow leads to a rise in Tmr and local ambient temperatures [7][13]. Taking these factors into account , these dynamic shifts directly alter the heat exchange equilibrium at the boundary surface (Tcl), which is viewed as a primary driver of the sudden onset of heat stress in crowded transit environments [16][27][34][65].

Localized Vapor Pressure (pvap,local):

pvap,local=pvap,env⋅1+η⋅D⋅MtMtrain

This formulation identifies the "Latent Heat Barrier" within the vehicle cabin. By calculating pvap,local, the model accurately reflects the suppression of evaporative heat loss (Esk) in crowded carriages. This imbalance triggers a surge in skin wettedness (ω), eventually reaching the 0.25 critical threshold and leading to the non-linear collapse of the psychological offset (λ).

Ambient Vapor Pressure (pvap,env)

This term serves as the thermodynamic baseline for the calculation. It is derived from macroscopic relative humidity (ϕrel) and ambient temperature (Text) consistent with the ISO 7726 standard [25][30][31][56].

Moisture Emission Linkage (η)

The model proposes the coefficient η≈0.022 to account for the mass fraction of water vapor within exhaled breath [50][52]. Consistent with the extreme pressure test boundary (D=9 pass/m2) specified by GB 50157-2013 [34], this value is determined by the following derivation [20][22][23]:

Assuming a steady metabolic state where MtMtrain=1 , the coefficient is calculated to ensure:

1 + η⋅9 = 1.20 ⇒η≈0.022

This calibration ensures that the localized vapor pressure is viewed as approximately 20% higher than the ambient average at maximum occupant density, thereby accurately reconstructing the moisture spikes observed in empirical literature [14][20][62][63][74].

Metabolic-Density Coupling (D⋅Mt)

The humidity spike is driven by the interaction between occupant density (D) and the transient metabolic trajectory (M(t)) [14]. As passengers exert higher energy—for instance, during rapid entry or while maintaining stability [13][20]—the intensified respiration further elevates the localized vapor pressure gradient [12][26][48][52].

Modeling Result

Observation and Discussion.

i. Observation and Discussions (15%) — 深度分析与“避坑”指南

这一部分是你的核心，不要只放图表，要解释“为什么模拟结果和直觉（或实测）有偏差”。

- 1. 建立基准（Baseline Validation）：

首先肯定 Fanger 模型的地位。你可以写道：“基于 ISO 7730 标准的 Fanger 稳态模型为地铁车厢热舒适度提供了物理学上最稳健的基准（Standardized Benchmark）。它在计算效率和跨平台标准化方面具有无可比拟的优势。”（这解释了为什么你选它：为了降低不必要的建模复杂度）。

- 2. 讨论“工作量 vs 准确度”的权衡（The Trade-off）：

直接指出：虽然生理参数（如 $M$）和辐射参数（如 $T_{mr}$）存在高度非线性和个体差异，但在初步设计阶段，采用标准推荐值（Standard values）可以快速锁定空调系统的热负荷区间。你可以讨论：如果引入更复杂的适应性模型，虽然准确度提升，但会增加传感器布点成本和计算延迟。

- 3. 针对“地铁拥挤场景”的特异性讨论（Metro-specific Distortion）：

这是最出彩的地方。你可以讨论在模拟中发现的现象：

  - 屏蔽效应： 讨论当乘客密度增加时，原本通过辐射散发给车窗的热量如何被周围乘客“截断”。

  - 代谢漂移： 提到站立乘客在列车加速时为了维持平衡，其实际 $M$ 值可能高于手册中的 1.2 met，这解释了为什么高峰期空调开得很足，乘客依然感到闷热。

Conclusion

结论部分要简练，给你的研究定性。

- 1. 模型定位：

总结该建模工作实现了一个高度模块化的稳态热平衡求解器，能够有效预测常规工况下的乘客热感觉。

- 2. 诚实面对局限（Limitations）：

用一句话带过：”本研究意识到经典 PMV 模型在处理极高人群密度和瞬态代谢变化时的局限性，其预测结果在极端拥挤工况下倾向于保守（偏乐观）。”

- 3. 提出改进路径（Future Work）：

“未来的优化方向可以是在本程序的物理核心基础上，引入适应性修正系数（e-coefficient）或针对亚洲人群代谢特征的性别加权参数，从而在不显著增加计算量的基础上，实现更高精度的热环境控制。”

Task 2

Modeling Result

Numerical Solution

Regarding the computational approach, the equation includes a quartic term ($T^4$), rendering it highly non-linear. Therefore, the Newton-Raphson iteration method is recommended to ascertain the veracity of the numerical convergence.

在你的程序循环中，方程二的执行逻辑如下：

- 输入接收：从环境端获取 $T_{ext}$ 和 $T_a$，通过热阻公式算得车窗/壁面的 $T_{wall,i}$。

- 生理耦合：从生理端获取当前步长的 $T_{cl}$（衣服表面温度）。

- 几何修正：代入由密度 $D$ 决定的动态角系数 $F_{p-p}$ 和 $F_{p-wall}$ 。

- 最终求解：通过上述细化方程求得 $T_{mr}'$，并将其反馈回 PMV 求解器 。

这种细化方式不仅保留了物理严谨性，还通过 $T_{ext} \rightarrow T_{wall,i} \rightarrow T_{mr}' \leftarrow T_{cl}$ 的链路，实现了辐射端与另外两端的全参数联动。接下来需要我为你处理 方程三（有效对流系数 $h_{cv,eff}$） 的细化定义吗？

Observation and Discussion

Conclusion

Nomenclature

HVAC:

MAPS: Metabolic Adjustment for Postural Stability

MRT: Mean Radiant Temperature

PMV: Predicted Mean Vote

DTL: Dynamic Thermal Load

SET: Standard Effective Temperature

M：Metabolic Rate

W：External Mechanical Work

R：Radiative Heat Exchange

C：Convective Heat Exchange

Esk ：Total Evaporative Heat Loss from Skin

Cre ：Sensible Respiratory Heat Loss

Ere ：Latent Respiratory Heat Los

S: Rate of Heat Storage

Pa : Ambient Vapor Pressure

Ta:  Ambient Temperature

τ：Time Constant

EATM: Expectancy-Adjusted Thermal Model

PMVdynamic : Dynamic Physical Base PMV

PMVEATM : Expectancy-Adjusted PMV

λ(t,THI, ω): Psychological Offset

L(t): Dynamic Thermal Load

M(t): Transient Metabolic Trajectory

Mplatform:  Initial Locomotive Metabolic Rate

Mtrain: Asymptotic Steady-State Metabolic Rate

Tphys: Physiological Time Constant

ω：Skin Wettedness

Esk: Actual Evaporative Heat Loss

Emax: Maximum Evaporative Capacity

Ediff: Insensible Perspiration

Qsw: Active Sweat Flux

Esw: Active Sweat Evaporation Heat Loss

hc: Convective Heat Transfer Coefficient

he: Evaporative Heat Transfer Coefficient

t: Exposure Time

ΔSETn-1: Coupled Magnitude

τexp: Temporal Decay Constant

RH: relative humidity

Tmr : Mean Radiant Temperature, ℃

Tmr': Corrected Mean Radiant Temperature, ℃

Va: Air Velocity

Icl: Clothing Insulation

Φ(t): Temporal Dissipation

τhab: Psychological Habituation Constant

G: Linear Expectancy Gain

fdef: Non-linear Boundary Defense Filter

ωcri： Critical Threshold of Wettedness

Fp-p: Human-to-Human View Factor

Fp-wall: Human-to-Wall View Factor

Fwall,0: Initial Baseline View Factor

kω: Wall Occlusion Decay Constant

kp : Interpersonal Saturation Rate

D: Occupant Density

Tmr': Corrected Mean Radiant Temperature,  ℃

Fi：Dynamic View Factor

fclo: Clo-Compression Function

Icl,final：Corrected Clothing Insulation

Icl,0：Basic Clothing Insulation

fcloD：Clo-Compression Function

fwetω：Moisture-dependent Insulation Correction Factor

αde: Decay Constant

fvent: Local Convective Shielding Factor

hcv,eff: Effective Convective Heat Transfer Coefficient

hcv,ref: Reference Convective Heat Transfer Coefficient

V: Nominal Air Velocity， m/s

ε: Numerical Stability Constant, m/s

γ: Flow Field Attenuation Coefficient, m2/pass

Σbsa: Total effective heat exchange surface area,  m2

mp: Individual mass of the passenger, kg

lp: Height of the passenger, cm

pvap:  Ambient Vapor Pressure, kPa

ϕrel: Macroscopic Relative Humidity

Text: External Ambient Air Temperatur

Tcl: Clothing Surface Temperature,  ℃

hc: Convective Heat Transfer Coefficient (W/m2⋅K)

hr: Radiative Heat Transfer Coefficient

Tmr: Mean Radiant Temperature,  ℃

Twall: Boundary Wall Temperature,  ℃

Text: External Ambient Temperature,  ℃

Fi: Dynamic View Factor

Vlocal：Local Air Velocity， m/s$

fvent: Local Convective Shielding Factor

Pvap,local: Localized Vapor Pressure, kPa

η: Moisture Emission Linkage Coefficient

ϕ: Metabolic-Density Coupling Factor

Reference

- China Urban Rail Transit Association. (2024, June). China urban rail transit integrated rail development guide.

- Beijing Transportation Research Center. (2012, August). 2012 Beijing transport development annual report.

- Sui, X., Chen, S., Han, B., & Yu, S. (2023). Evaluation of the thermal environment of Xi’an subway stations in summer and determination of the indoor air design temperature for air-conditioning. Sustainability, 15(7), 6111. https://doi.org/10.3390/su15076111

- Borgi, M., Ghasemi, K., Tasnim, S. H., & Mahmud, S. (2023). Investigation of thermal comfort conditions by heat pump hybridized with phase change material-based solar desiccant cooling system. Applied Thermal Engineering, 232, 121097. https://doi.org/10.1016/j.applthermaleng.2023.121097

- Patel, V. K., Boudreaux, P. R., & Gluesenkamp, K. R. (2020). Validated model of a thermoelectric heat pump clothes dryer using secondary pumped loops. Applied Thermal Engineering, 184, 116345. https://doi.org/10.1016/j.applthermaleng.2020.116345

- Cui, Y., Fan, C., Zhang, W., Zhou, X., & Li, J. (2024). Climate change's effects on the amount of energy used for cooling in hot, humid office buildings and the solutions. Journal of Cleaner Production, 442, 140967. https://doi.org/10.1016/j.jclepro.2024.140967

- Liu, Z., Long, Y., Yang, T., Zhu, Y., Liu, M., Li, P., & Li, H. (2023). Research on air supply volume and temperature control of subway trains based on passenger capacity. Atmosphere, 14(2), 258. https://doi.org/10.3390/atmos14020258

- Buonomano, A., Forzano, C., Giuzio, G. F., Palombo, A., & Russo, G. (2024). Energy efficiency and indoor thermal comfort of railway carriages: Development of an innovative passenger-centric-control framework for HVAC systems. Energy, 307, 132440. https://doi.org/10.1016/j.energy.2024.132440

- Chen, S., Wang, H., Li, Q., & Song, Z. (2026). Application of machine learning-based thermal comfort model to optimize vehicle air-conditioning control strategies. Case Studies in Thermal Engineering, 77, 107578. https://doi.org/10.1016/j.csite.2025.107578

- Cuce, E., Riffat, S. B., & Young, C.-H. (2015). Thermal insulation, power generation, lighting and energy saving performance of heat insulation solar glass as a curtain wall application in Taiwan: A comparative experimental study. Energy Conversion and Management, 96, 31–38. https://doi.org/10.1016/j.enconman.2015.02.062

- EL-Massry, A. S., El-Negiry, E., Bekheit, M., & Soliman, A. S. (2025). Energy saving in rail vehicles air conditioning system using multiple phasechange materials. Thermal Science and Engineering Progress, 68, 104339. https://doi.org/10.1016/j.tsep.2025.104339

- Fang, Z., Zhang, P., Wu, P., Li, L., & Zheng, Z. (2023). Effects of different metabolic rates on thermal comfort in a warmer thermal environment. Journal of Building Engineering, 80, 108030. https://doi.org/10.1016/j.jobe.2023.108030

- Liu, J., Foged, I. W., & Moeslund, T. B. (2022). Automatic estimation of clothing insulation rate and metabolic rate for dynamic thermal comfort assessment. Pattern Analysis and Applications, 25, 619–634. https://doi.org/10.1007/s10044-021-00961-5

- Luo, M., Wang, Z., Ke, K., Cao, B., Zhai, Y., & Zhou, X. (2018). Human metabolic rate and thermal comfort in buildings: The problem and challenge. Building and Environment, 131, 44–52. https://doi.org/10.1016/j.buildenv.2017.12.033

- Mohammed, A. G., Hasini, H., Om, N. I., Elfeky, K. E., & Alhassan, A. B. (2026). Learning-based temperature prediction for advanced battery thermal management systems. Applied Thermal Engineering, 297, 130775. https://doi.org/10.1016/j.applthermaleng.2026.130775

- Ning, H., Wang, Z., Zhang, X., & Ji, Y. (2016). Adaptive thermal comfort in university dormitories in the severe cold area of China. Building and Environment, 99, 161–169. https://doi.org/10.1016/j.buildenv.2016.01.022

- Shu, X., Dong, Y., Liu, J., & Xu, X. (2024). Study of the optimal control of the central air conditioning cooling water system for a deep subway station in Chongqing. Buildings, 15(1), 8. https://doi.org/10.3390/buildings15010008

- State Administration for Market Regulation & Standardization Administration of the People's Republic of China. (2021). Guidelines for the control of certain chemical substances in clothing accessories and components (GB/T 40228—2021).

- Wei, D., Yang, L., Bao, Z., Lu, Y., & Yang, H. (2022). Variations in outdoor thermal comfort in an urban park in the hot-summer and cold-winter region of China. Sustainable Cities and Society, 77, 103535. https://doi.org/10.1016/j.scs.2021.103535

- Yu, Y., You, S., Zhang, H., Ye, T., Wang, Y., & Wei, S. (2021). A review on available energy saving strategies for heating, ventilation and air conditioning in underground metro stations. Renewable and Sustainable Energy Reviews, 141, 132440

https://doi.org/10.1016/j.rser.2021.110788

- Zahid, M., & Faiz, K. (2026). CFD analysis of heat transfer in an air-cooled battery thermal management system. Khalifa Journal of Mechanical Research (KJMR), 3(3), 53–65. https://doi.org/10.71146/kjmr862

- Fanger, P. O. (1973). Assessment of man's thermal comfort in practice. British Journal of Industrial Medicine, 30(4), 313–324. https://doi.org/10.1136/oem.30.4.313

- Ministry of Housing and Urban-Rural Development of the People's Republic of China. (2013). Code for design of metro (GB 50157-2013). China Architecture & Building Press.

- State Administration for Market Regulation & Standardization Administration of the People's Republic of China. (2020). Test method for measuring the evaporative resistance of clothing—Sweating manikin method (GB/T 39605—2020). Standards Press of China.

- State Administration for Market Regulation & Standardization Administration of the People's Republic of China. (2021). Ergonomics of the thermal environment—Instruments for measuring physical quantities (GB/T 40233—2021/ISO 7726:1998). Standards Press of China.

- General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China & Standardization Administration of the People's Republic of China. (2008). Ergonomics of the thermal environment—Determination of metabolic rate (GB/T 18048—2008/ISO 8996:2004). Standards Press of China.

- General Administration of Quality Supervision, Inspection and Quarantine of the People's Republic of China & Standardization Administration of the People's Republic of China. (2017). Ergonomics of the thermal environment—Analytical determination and interpretation of thermal comfort using calculation of the PMV and PPD indices and local thermal comfort criteria (GB/T 18049—2017/ISO 7730:2005). Standards Press of China.

- Tartarini, F., Schiavon, S., Cheung, T., Hoyt, T., 2020. CBE Thermal Comfort Tool : online tool for thermal comfort calculations and visualizations. SoftwareX 12, 100563. https://doi.org/10.1016/j.softx.2020.100563

- Li, H., Hu, H., Wu, Z., Kong, X., & Fan, M. (2025). Modified predicted mean vote models for human thermal comfort: An ASHRAE database-based evaluation. Renewable and Sustainable Energy Reviews, 209, 115042. https://doi.org/10.1016/j.rser.2024.115042

- d’Ambrosio Alfano, F. R., Ianniello, E., & Palella, B. I. (2013). PMV–PPD and acceptability in naturally ventilated schools. Building and Environment, 67, 129–137. https://doi.org/10.1016/j.buildenv.2013.05.013

- Omidvar, A., & Kim, J. (2020). Modification of sweat evaporative heat loss in the PMV/PPD model to improve thermal comfort prediction in warm climates. Building and Environment, 176, 106868.

- Tartarini, F., & Schiavon, S. (2025). Comparative analysis of PMV Models accuracy implemented in the ISO Modified predicted mean:2005 and ASHRAE 55:2023. Building and Environment, 275, 112766.

- Wang, J., Wang, Y., Dong, Q., Yao, W., Sun, L., Zhao, H., & Fei, F. (2025). Short-term thermal perception and evaluation model under temperature step changes of building transition spaces. Building and Environment, 276, 112850.

- Shi, F., Li, N., Zheng, W., Song, M., Wang, M., & Cui, X. (2026). A thermal history-incorporated RWI model for dynamic thermal comfort in transportation-commercial integrated continuous underground spaces. Building and Environment, 296, 114494.

- Khadka, S., Rijal, H. B., Amano, K., Saito, T., Imagawa, H., Uno, T., Genjo, K., Takata, H., Tsuzuki, K., Nakaya, T., Nishina, D., Hasegawa, K., & Mori, T. (2024). Development of adaptive behaviour models for thermal comfort: Longitudinal investigation in Japanese office buildings and literature review. Journal of Building Engineering, 89, 109220.

- Humphreys, M. A., & Nicol, J. F. (1998). Understanding the adaptive approach to thermal comfort. ASHRAE Transactions, 104(1), 991–1004.

- Unknown Author. (2017). Effects of air temperature step changes on thermal perception and perceived arousal in an actual environment under hot-humid climate conditions. International Conference of Occupational Health and Safety 2017.

- Unknown Author. (n.d.). Adaptive thermal comfort explained by PMV.

- Humphreys, M. A. (n.d.). Chapter 10. Summary-paper prepared for the International Exchange Committee of the Kinki Branch of the Society of Heating, Air-Conditioning and Sanitary Engineers of Japan (SHASE).

- Yao, R., Zhang, S., Du, C., Schweiker, M., Hodder, S., Olesen, B. W., Toftum, J., d'Ambrosio, F. R., Gebhardt, H., Zhou, S., Yuan, F., & Li, B. (2022). Evolution and performance analysis of adaptive thermal comfort models – a comprehensive literature review. Building and Environment, 217, 109020.

- Wikipedia contributors. (2015). Thermal comfort. In Wikipedia, The Free Encyclopedia.

- Hou, G., Zhai, X., Kuai, Y., Shu, P., Zhang, P., & Shen, W. (n.d.). A systematic review on studies of thermal comfort in building transitional space. HeBei University of Technology / University College London.

- Rijal, H. B., Humphreys, M. A., & Nicol, J. F. (2015). [Development of an adaptive model using thermal comfort survey-data from Japanese dwellings]. Buildings, 5, 1037–1054.

- Li, R., Liu, Y., Yu, G., Guo, H., & Qin, S. (2023). Establishment of a thermal comfort model for spectator areas of air-supported membrane ice rinks in severe cold regions: A case study in Harbin, China. Energies, 16(12), 4598.

- Nguyen, A. T., Singh, M. K., & Reiter, S. (2012). An adaptive thermal comfort model for hot humid South-East Asia. Building and Environment, 56, 291–300.

- Cheung, T., Schiavon, S., Parkinson, T., Li, P., & Brager, G. (2019). Analysis of the accuracy on PMV – PPD model using the ASHRAE Global Thermal Comfort Database II. Building and Environment, 10.1016/j.buildenv.2019.01.055.

- Militký, J., Křemenáková, D., Kubeček, T., & Venkataraman, M. (2026). Indirect prediction of textile materials’ thermal insulation based on heat loss. Textiles, 6, 8.

- Jia, X., Li, S., Zhu, Y., Ji, W., & Cao, B. (2023). Transient thermal comfort and physiological responses following a step change in activity status under summer indoor environments. Energy and Buildings, 285, 112918. https://doi.org/10.1016/j.enbuild.2023.112918

- Li, R., Liu, J., Chen, X., Zhang, W., Lei, T., Chen, J., Xia, Y., & Bantserova, O. L. (2025). Transient thermal comfort during summer in air-conditioned indoor and naturally ventilated transitional spaces: A field study in Zhengzhou, China. Energy and Buildings, 328, 115122. https://doi.org/10.1016/j.enbuild.2024.115122

- Li, Y., Zhong, X., Xie, Y., de Dear, R., Lu, S., Lin, B., & Niu, J. (2026). Thermal comfort on the move: Understanding thermal alliesthesia on urban walking routes. Building and Environment, 295, 114434. https://doi.org/10.1016/j.buildenv.2026.114434

- Xu, Z., Zhao, N., Yan, Y., Gao, S., & Hillmansen, S. (2025). Electric-thermal collaborative system and control for hydrogen-fuel cell passenger trains in the UK’s winter. Energy Conversion and Management, 328, Article 119629. https://doi.org/10.1016/j.enconman.2025.119629

- Wieser, S., Naveau, L., Kraft, W., Beristain, R. A., & Anson, A. (2025). Modeling and simulation of thermal management in battery electric rail vehicles for the sizing of thermal storage systems. Energy Conversion and Management, 346, Article 120489. https://doi.org/10.1016/j.enconman.2025.120489

- Lu, S., Li, Z., Zhao, Q., & Jiang, F. (2017). Modified calculation of solar heat gain coefficient in glazing façade buildings. Energy Procedia, 122, 151–156. https://doi.org/10.1016/j.egypro.2017.07.335

- Zheng, W., Wen, X., Cai, J., Li, C., & He, D. (2021). Characteristics of thermal environment influenced by the opening and closing of a subway train sliding door. Case Studies in Thermal Engineering, 27, Article 101278. https://doi.org/10.1016/j.csite.2021.101278

- Song, J., Zhang, Y., Huang, Y., Wang, P., & Yang, B. (2024). Ventilation strategy for particulate matter control in subway stations. Journal of Building Engineering, 98, Article 111323. https://doi.org/10.1016/j.jobe.2024.111323

- Gao, Y., Meng, L., Li, C., Ge, L., & Meng, X. (2023). An experimental study of thermal comfort zone extension in the semi-open spray space. Developments in the Built Environment, 15, Article 100217. https://doi.org/10.1016/j.dibe.2023.100217

- Zhou, W.-Y., Zhang, X.-Y., & Lu, M. (2024). Electromagnetic exposure analysis of the subway passenger under the civil communication system radiation. PLoS ONE, 19(3), Article e0300049. https://doi.org/10.1371/journal.pone.0300049

- Wu, X., Ling, R., Wan, X., Ren, H., Jing, X., & Feng, G. (2025). Study on the Impact of Ventilation Methods on Droplet Nuclei Transmission in Subway Carriages. Applied Sciences, 15(9), Article 4919. https://doi.org/10.3390/app15094919

- Mjörnell, K., Johansson, D., & Bagge, H. (2019). The Effect of High Occupancy Density on IAQ, Moisture Conditions and Energy Use in Apartments. Energies, 12(23), Article 4454.

- Wen, Y., Leng, J., Shen, X., Han, G., Sun, L., & Yu, F. (2020). Environmental and Health Effects of Ventilation in Subway Stations: A Literature Review. International Journal of Environmental Research and Public Health, 17(3), Article 1084.

- Joshi, A., Viswanathan, S. H., Jaiswal, A. K., Sadeghi, K., Bartels, L., Jain, R. M., Pathikonda, G., Vanos, J. K., Middel, A., & Rykaczewski, K. (2024). Characterization of human extreme heat exposure using an outdoor thermal manikin. Science of the Total Environment, 923, 171525. https://doi.org/10.1016/j.scitotenv.2024.171525

- Zhang, P., Wu, F., Wang, H. S., Su, X., Gao, G., & Lu, W. (2025). Fiala model-based evaluation of metro thermal comfort and energy performance under impinging jet ventilation. Energy and Buildings, 349, 116607. https://doi.org/10.1016/j.enbuild.2025.116607

- Zhou, Y., & Zheng, X. (2026). Manikin-involved CFD modeling and applications for air flow analysis: A comprehensive review. Renewable and Sustainable Energy Reviews, 229, 116561. https://doi.org/10.1016/j.rser.2025.116561

- Ozeki, Y., Konishi, M., Narita, C., & Tanabe, S. (n.d.). Effective radiation area of human body calculated by a numerical simulation. AIVC 12070.

- Wu, F., Huang, Y., Pan, X., Lu, J., Xu, R., Yu, C., & Lin, X. (2025). The characteristics of passenger heat loss and thermal environment in a subway cabin. Physics of Fluids, 37(3), 035141. https://doi.org/10.1063/5.0256344

- He, N., & Li, G. (2021). Urban neighbourhood environment assessment based on street view image processing: A review of research trends. Environmental Challenges, 4, 100090. https://doi.org/10.1016/j.envc.2021.100090

- Jeong, J., Lee, I., Oh, Y., Park, J., & Ahn, S. (2025). Assessment of impact of bottlenecks on evacuation in subway stations using network analysis. Developments in the Built Environment, 21, 100632. https://doi.org/10.1016/j.dibe.2025.100632

- Kong, X., Wang, Z., Fan, M., & Li, H. (2023). Analysis on the energy efficiency, thermal performance and infection intervention characteristics of interactive cascade ventilation (ICV). Journal of Building Engineering, 68, 106045. https://doi.org/10.1016/j.jobe.2023.106045

- Li, Q., Cui, C., Liu, F., Wu, Q., Run, Y., & Han, Z. (2022). Multidimensional urban vitality on streets: Spatial patterns and influence factor identification using multisource urban data. ISPRS International Journal of Geo-Information, 11(1), 2. https://doi.org/10.3390/ijgi11010002

- Mba, E. J., Oforji, P. I., Okeke, F. O., Ozigbo, I. W., Onyia, C. D. F., Ozigbo, C. A., Ezema, E. C., Awe, F. C., Nnaemeka-Okeke, R. C., & Onyia, S. C. (2025). Assessment of floor-level impact on natural ventilation and indoor thermal environment in hot–humid climates: A case study of a mid-rise educational building. Buildings, 15, 686. https://doi.org/10.3390/buildings15050686

- Middel, A., Lukasczyk, J., Maciejewski, R., Demuzere, M., & Roth, M. (2018). Sky View Factor footprints for urban climate modeling. Urban Climate, 25, 120–134. https://doi.org/10.1016/j.uclim.2018.05.004

- Wang, H., Jiang, Z., Xu, T., & Li, F. (2021). A quantitative approach of subway station passengers’ heterogeneity of decision preference considering personality traits during emergency evacuation. Sustainability, 13(22), 12540. https://doi.org/10.3390/su132212540

- Wang, M., & Chen, Q. (2010). On a hybrid RANS/LES approach for indoor airflow modeling. HVAC&R Research.

- Wang, Q., Wu, L., Jiang, Y., & Xu, Z. (2025). A study of thermal environment patterns in subway cars and the influence of age on passenger comfort. Case Studies in Thermal Engineering, 72, 106268. https://doi.org/10.1016/j.csite.2025.106268

- Wolkoff, P. (2024). Indoor air humidity revisited: Impact on acute symptoms, work productivity, and risk of influenza and COVID-19 infection. International Journal of Hygiene and Environmental Health, 256, 114313. https://doi.org/10.1016/j.ijheh.2023.114313

- Wu, F., Yu, C., Xu, R., & Li, H. (2024). Numerical simulation of droplet dispersion and passenger infection risk assessment in a subway carriage. Applied Sciences, 14(2), 590. https://doi.org/10.3390/app14020590

Appendix

| 符号 | 学术名称 (Nomenclature) | 单位 | 物理意义与建模说明 |
| --- | --- | --- | --- |
| $h_{cv,eff}$ | Effective Convective Heat Transfer Coefficient | $W/(m^2 \cdot K)$ | 考虑人群屏蔽效应后，到达乘客体表的实际对流换热系数 。 |
| $h_{cv,ref}$ | Reference Convective Heat Transfer Coefficient | $W/(m^2 \cdot K)$ | 基于标准风速 $V$ 计算的参考对流换热系数（通常采用 ISO 7730 标准公式）。 |
| $V$ | Nominal Air Velocity | $m/s$ | 暖通系统（HVAC）提供的宏观送风风速 。 |
| $D$ | Occupant Density | $pass/m^2$ | 车厢内的实时乘员密度 。 |
| $\gamma$ | Flow Field Attenuation Coefficient | $(m/s) \cdot (m^2/pass)$ | 流场衰减系数：量化人群对气流的物理阻滞能力。 |
| $\epsilon$ | Numerical Stability Constant | $m/s$ | 稳定性因子：通常设定为 $0.001$，用于消除 $V=0$ 时的奇点，并表征极低风速下的自然对流底线。 |